# 🌍 Flood Mapping Dataset Preparation (Ibadan, Nigeria)

### 1. Data Collection

- Satellite data: Sentinel-2 (from Sentinel Hub EO Browser).

- Case study images:

  - Dry season: Dec 01, 2023 – Feb 29, 2024 → 1 dataset.

  - Rainy season: Jul 15 – Jul 25, 2023 → 1 dataset.

- Seasonal series (generalization):

  - Dry season: 5 datasets (Dec 2023 – Feb 2024).

  - Rainy season: 9 datasets (Apr 2023 – Oct 2023).

- Terrain data: SRTM DEM (30 m) from USGS EarthExplorer.

### 2. Preprocessing Sentinel-2 Data in QGIS

- Build Virtual Raster (VRT) → Raster → Miscellaneous → Build VRT (resampling = Nearest Neighbour).

- Convert to GeoTIFF → Raster → Conversion → Translate.

- Clip to study area → Raster → Extraction → Clip by Mask Layer.

- ✅ Output: Clipped Sentinel-2 stacks for all case study + seasonal images.

### 3. Spectral Index Computation (Raster Calculator)

Indices derived from Sentinel-2 stacks:

- NDVI = (B08 – B04) / (B08 + B04) → ibadan_ndvi.tif

- NDWI = (B03 – B08) / (B03 + B08) → ibadan_ndwi.tif

- MNDWI = (B03 – B11) / (B03 + B11) → ibadan_mndwi.tif

- NDBI = (B11 – B08) / (B11 + B08) → ibadan_ndbi.tif

### 4. Water Mask Generation

- Thresholding (on MNDWI):

- mndwi > 0.2

→ 1 = Water, 0 = Non-water → ibadan_mask_raw.tif.

- Cleaning: Raster → Miscellaneous → Sieve → ibadan_mask_clean.tif.

- Alignment: Raster → Projections → Align Rasters (reference = Sentinel-2 B04 or B08) → aligned.tif.

- Manual editing: Case study masks refined for accuracy.

### 5. DEM-Based Terrain Analysis

Using SRTM DEM (30 m) in QGIS (SAGA / NextGen tools):

- DEM → ibadan_dem.tif

- Slope → ibadan_slope.tif

- Specific Catchment Area (SCA) → ibadan_sca.tif

- Topographic Wetness Index (TWI) → ibadan_twi.tif

These describe hydrological/topographic controls on flooding.

### 6. Dataset Organization

📂 Sentinel_stacks/

ibadan_2023_dry_stack.tif

ibadan_2023_rainy_stack.tif

scene1_dry_stack.tif … scene9_rainy_stack.tif

📂 Masks/

ibadan_2023_dry_label.tif

ibadan_2023_rainy_label.tif

scene1_dry_label.tif … scene9_rainy_label.tif

📂 DEM_layers/

ibadan_dem.tif

ibadan_slope.tif

ibadan_sca.tif

ibadan_twi.tif

### 7. Verification

- Overlaid Sentinel-2 stacks + masks in QGIS.

- Used transparency/false-color composites to check pixel-level alignment.

- ✅ Confirmed flood pixels (blue) match water bodies in Sentinel-2 imagery.

### 👉 Summary

You:

- Collected Sentinel-2 + DEM data.

- Preprocessed & clipped Sentinel-2 stacks.

- Computed vegetation, water, and built-up indices.

- Generated & cleaned flood masks (binary).

- Aligned masks to Sentinel-2 reference grids.

- Derived DEM-based hydrological layers (slope, SCA, TWI).

- Organized into a structured dataset (stacks, labels, DEMs).

- Verified alignment and accuracy in QGIS.

The dataset is now ready for machine learning–based flood detection and risk modeling.

✨ Question: Do you want me to also add a flowchart diagram of this pipeline (from Sentinel-2 → preprocessing → indices → masks → DEM → final dataset)

big picture goal of this dataset preparation:

- Sentinel-2 stacks (multi-band input) + Masks (flood/no-flood)
→ You clipped, aligned, and prepared them.

- Patch extraction
→ Split large rasters (e.g. 3000×3000 px) into smaller CNN-ready tiles (e.g. 256×256).
This makes it computationally feasible.

- Train/Val/Test splits
→ Organize tiles into splits for model training.

- PyTorch Dataset & DataLoader
→ So the model can batch-load (image, mask) pairs, apply augmentation, and train.

# Summary of Model Development and Training

- Dataset Preparation

  - Sentinel-2 satellite imagery was used, focusing on relevant spectral bands (Blue, Green, Red, NIR, SWIR1, SWIR2).

  - Binary flood masks were prepared to label flooded (1) and non-flooded (0) areas.

  - Patches of images and masks were extracted and organized into training and validation sets.

- Data Balancing

  - Since flood pixels are much fewer than background pixels, a WeightedRandomSampler was applied.

  - This ensures the model sees a balanced number of flood and non-flood samples during training, preventing bias toward predicting only background.

- Data Normalization

  - Because Sentinel-2 data are not natural RGB images, standard ImageNet statistics were not used.

  - Instead, pixel values were normalized between 0 and 1 using A.Normalize(mean=0.0, std=1.0, max_pixel_value=1.0).

  - This prevents large pixel intensity differences from affecting training stability.

- Model Definition

  - A U-Net segmentation model was implemented using the ResNet-18 encoder (via the segmentation_models_pytorch library).

  - The input channels were set to 6 (corresponding to the selected Sentinel-2 bands).

  - The output was 1 channel for binary classification (flood vs background).

  - No ImageNet pretrained weights were used (encoder_weights=None) because Sentinel imagery is different from natural images.

- Loss Function

  - A combined Binary Cross-Entropy (BCE) Loss + Dice Loss was used.

  - BCE handles pixel-wise classification, while Dice emphasizes overlap between predicted floods and ground truth.

  - Class imbalance was addressed by setting a positive class weight (e.g., 5× more weight on flood pixels).

- Evaluation Metrics

  - Intersection over Union (IoU) and F1-Score were computed using the torchmetrics library.

  - IoU measures how well predicted flood areas overlap with ground truth.

  - F1-Score balances precision (avoiding false floods) and recall (detecting actual floods).

- Optimizer and Scheduler

  - The Adam optimizer with a learning rate of 0.0001 was used.

  - A ReduceLROnPlateau scheduler was included to reduce the learning rate automatically when validation performance stagnates.

- Training Loop

  - The model was trained for 100 epochs.

  - In each epoch:

    - The model learned from training patches using weighted sampling.

    - Validation was performed after each epoch, and IoU/F1 were recorded.

    - The best model checkpoint was saved automatically when validation IoU improved.

Still Continue
…..